Digital Logic Design Prqactice Questions

MCQ’s

- The binary equivalent of 0.15510 is

  - 0.000001

  - 0.001011

  - 0.001000

  - 0.001001

- The binary equivalent of 7018 is

  - 111000001

  - 111000000

  - 110000001

  - 111100001

- The 8-bit signed binary equivalent of -2810 is

  - 1111 0100

  - 1110 0100

  - 1110 1100

  - 1111 0100

- The octal equivalent of signed 8-bit binary number 1111 11002 is

  - 274

  - 374

  - 774

  - 674

- Applying the Boolean algebra techniques to simplify the expression AB + A(B + C) + B(B + C), the resultant simplified expression is:

  - B + AC

  - AB + B

  - B + BC

  - C + AB

- Using the DeMorgan’s theorem to simplify F=X.Y.(Y+Z), the resultant simplified expression is:

  - F=XY+YZ

  - F=XY+YZ

  - F=XY+YZ

  - F=XY+YZ

- Given the following Karnaugh map,  the minimum Boolean POS expression is:

| CD\AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 1 | 0 |
| 01 | 1 | X | X | 1 |
| 11 | X | 1 | 1 | X |
| 10 | 0 | 0 | 1 | 0 |

  - (A+C)(B+C)

  - (A+D)(B+D)

  - (B+A)(B+D)

  - (A+D)(C+D)

- A circuit has four inputs (A,B,C,D) representing a single decimal number ranging from 0 to 15 and an output (Y) that denotes whether the decimal number is divisible by 5. The SOP canonical of Y is:

  - Y= (0,5,10,15)

  - Y= (0,3,10,15)

  - Y= (0,5,14,15)

  - Y= 0,5,13,15

Mixed

- Convert

- 12.4510 to Base 2

- Convert 12.416 to Base 10

- What range of hexadecimal numbers fit in 4 bytes field

- Using 2’s complement conversion on operands, perform the following operations. Show all working in binary 8-bit numbers.

- Compute the following using two’s complement

1100 11112 – 1100 00002
01110001

- A four-bit binary number {A,B,C,D}, where A is the most significant digit and D the least significant digit, appears on the input to a combinational logic circuit. Output X indicates whether the number is divisible by 2 without any remainder and output Y indicates if the number is divisible by 3 without remainder. Obtain the sum-of-products logic equations for X and Y, simplify using K-Maps and draw the logic diagram.